VESPER-SEC-01 — Week 7 Progress Report

Project: Cryptographic Primitive Benchmarking Under Cycle-Accurate Emulation for Nano-Bee Secure Communications
Domain: Cybersecurity (Option 3.5 A)
Subsystem: B-System · Mothership
Reporting period: Week 7 — Extension
Author: Rutvika Mahadev Ashtekar

1. What Was Attempted

Week 7 extends the Week 6 ASCON benchmark from a single 16-byte input case to four input sizes: 16, 64, 256 and 1,024 bytes. The benchmark uses the existing ASCON reference implementation, a Cortex-M4-targeted test ELF and a Renode machine named ascon7.

The program records a cycle-counter value for each encryption, checks the encryption return code and expected ciphertext length, and sets completion and validity flags. The purpose of this extension is to examine the benchmark across different message sizes without changing Week 6 results.

2. What Worked

- The ascon_benchmark_test.elf image loaded and executed in the recorded Renode session.

- The completion flag, result_done, returned 1.

- All four validity flags, at 0x20000008, 0x2000000C, 0x20000010 and 0x20000014, returned 1.

- Four non-zero values were read from the addresses allocated to result_cycles.

- The ELF section inspection confirmed .bench_buffers at 0x20010000, with a section size of 0x810 bytes.

- A clean Renode screenshot records the setup, flags and readings.

These checks demonstrate execution and successful output-length checks. They do not independently validate the accuracy of the cycle measurements.

3. Preliminary Observations

| Input size | First recorded run | Subsequent recorded run |
| --- | --- | --- |
| 16 bytes | 10,903 cycles | 10,903 cycles |
| 64 bytes | 13,813 cycles | 13,813 cycles |
| 256 bytes | 38,650 cycles | 25,684 cycles |
| 1,024 bytes | 137,998 cycles | 150,964 cycles |

The 16-byte and 64-byte readings matched between the two recorded runs. The 256-byte and 1,024-byte readings did not. Therefore, these values are retained as preliminary observations, not reproducible or validated timing results.

The firmware disables the DWT counter before setting result_done. A post-execution read returned DWT_CTRL = 0 and DWT_CYCCNT = 0x00049D40. These post-execution values do not establish that the four recorded measurements are accurate.

4. What Did Not Work / Open Items

- Repeatability of the longer-message measurements remains unresolved.

- The successful Renode configuration has been reconstructed from the recorded commands, but a saved, independently replayed setup has not yet been demonstrated.

- The exact source-to-ELF build relationship has not been conclusively verified. The source and test ELF have different modification timestamps, and no ASCON build script was found in the inspected Week 7 directories.

- The DWT timing method has not been validated against a comparable published cryptographic result.

- No final cycles-per-byte, energy, or resource-feasibility conclusion is drawn from these preliminary readings.

5. Changes to the Plan

No Week 6 files or recorded results were changed. The Week 7 work remains focused on extending the ASCON benchmark to multiple message sizes. The observed discrepancy will be documented and investigated rather than resolved by selecting or averaging measurements without evidence.

6. Next Week's Specific Objective

Week 8 is the validation stage. The next objective is to investigate measurement repeatability, validate the timing method against a genuinely comparable published result, record the discrepancy, and investigate the emulation configuration if the discrepancy exceeds the project's stated tolerance. Any unresolved limitation will remain explicit. No final cryptographic feasibility verdict will be made from unvalidated timing data.

7. Evidence and Reproducibility

Evidence currently available includes the ASCON source, test ELF, map file, linker configuration, Renode command sequence, memory-read outputs and the benchmark screenshot. The runtime flags and cycle readings were observed in the recorded session. A saved replay script and independent confirmation of reproducible timing remain outstanding.

Status: Week 7 extension implemented and executed; preliminary output checks passed. Reproducibility and cryptographic timing accuracy remain open verification items.